# SIMATS ENGINEERING

Department of Computer Science and Engineering

## ASSESSMENT TOOL 1 – INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 – Precision (BL4) | Assessment: | Industry Problem solving task |
| Weightage: | 30% | Total Marks: | 100 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | K.Rekanth | Reg. No.: | 192311213 |
| Section / Batch: | CSE | Date: | 18-08-2026 |

Instructions to Students

- Identify the relevant concepts of cache hierarchy, SRAM, DRAM, MRAM, NAND Flash, RRAM, memory latency, bandwidth, and virtual memory paging.

- Analyze the memory-access requirements of smartphones, cloud servers, autonomous vehicles, edge AI devices, and gaming consoles, considering latency, bandwidth, capacity, and power consumption.

- Apply suitable memory-hierarchy techniques such as cache optimization, appropriate memory technology selection, data locality, paging optimization, and hierarchical memory organization.

- Compute performance measures such as memory access time, latency, bandwidth, throughput, speedup, hit/miss rates, and resource or power utilization where applicable.

- Interpret the results by evaluating performance improvements, memory bottlenecks, technology trade-offs, and the suitability of the proposed memory hierarchy for each application.

QUESTIONS

- A mobile device manufacturer observes noticeable lag when users switch rapidly between multiple applications. Analyze how L1, L2, and L3 caches and DRAM participate in storing and retrieving frequently accessed application data. Based on latency, capacity, hit rate, and bandwidth requirements, propose a suitable hierarchical memory organization that can reduce application-switching delays and improve overall system throughput.

- A cloud service provider experiences increased latency when processing database queries from a large number of concurrent users. Analyze how cache hierarchy and virtual memory paging influence memory access performance, and compare their roles in reducing processor-memory latency. Recommend suitable memory technologies and memory-management improvements that can minimize page faults, reduce access delays, and improve database throughput.

- An autonomous vehicle continuously receives large volumes of sensor data from cameras, LiDAR, radar, and other real-time sensing systems. Evaluate the characteristics of SRAM, DRAM, and MRAM in terms of access latency, bandwidth, capacity, volatility, and power consumption. Based on the real-time processing requirements, design an appropriate memory hierarchy that can provide high data-transfer bandwidth while minimizing access latency and supporting reliable sensor-data processing.

- An edge-based AI device must load machine-learning models quickly while operating under strict power and storage constraints. Analyze the characteristics of NAND Flash, DRAM, and RRAM with respect to access speed, storage capacity, persistence, power consumption, and suitability for AI workloads. Recommend a hierarchical memory organization that enables fast model loading, efficient intermediate-data processing, reduced energy consumption, and improved overall inference performance.

- A high-performance gaming console experiences frame drops and noticeable delays when loading large game environments containing high-resolution textures, audio, and other assets. Analyze the interaction between L3 cache and DRAM during the retrieval and processing of frequently accessed game data, considering their latency, capacity, and bandwidth. Propose suitable memory-hierarchy enhancements that can reduce data-access bottlenecks, increase memory throughput, and provide smoother frame rendering during large scene transitions.

Code of Conduct

I K. Rekanth(192311213)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

## Signature of the Student: K. Rekanth

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Industry Problem | 20% | Comprehensive understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding; some requirements unclear | Poor understanding; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementation | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |
| Professionalism | 5% | Highly professional, well-documented, and clearly presented work | Good documentation and presentation | Basic documentation with some gaps | Poor documentation and presentation |

## Marks Evaluation:

| Q. No | Understanding of Industry Problem (20) |  | Application of Engineering Concepts (25) |  | Solution Design (25) | Use of Tools (15) | Analysis (10) | Professiona lism(5) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 25 |  | / 30 |  | / 20 | / 15 | / 10 | /5 | / 100 |

Course Coordinator	Faculty Incharge

- Mobile Device – Cache Hierarchy and DRAM

L1 cache is the smallest and fastest cache and stores frequently accessed instructions and data. L2 is larger but slower than L1, while L3 is larger and generally shared by processor cores. DRAM provides much larger capacity but has higher latency than cache memory.

• L1: Store the most frequently used application instructions and data close to the CPU.

• L2: Hold a larger working set when required data is not available in L1.

• L3: Provide a shared cache for data used by multiple CPU cores.

• DRAM: Hold application working sets that cannot fit into the cache hierarchy.

Recommended hierarchy: CPU registers → L1 cache → L2 cache → shared L3 cache → DRAM → storage. Cache-friendly data locality and suitable prefetching can improve the hit rate.

Result: Increasing cache hit rates reduces accesses to slower DRAM, reducing application-switching delay and improving overall throughput.

2. Cloud Database – Cache Hierarchy and Virtual Memory

Cache hierarchy reduces processor-memory latency by keeping frequently accessed database instructions and data close to the CPU. Virtual memory paging maps virtual pages to physical memory and allows processes to use a larger logical address space.

• Use L1, L2 and L3 caches for frequently accessed database instructions and hot data.

• Provide sufficient DRAM so active database pages remain in physical memory.

• Reduce page faults because a page fault can require much slower secondary-storage access.

• Use working-set management, suitable page replacement and efficient memory allocation.

• Use high-performance storage for unavoidable paging and database persistence.

Recommended approach: optimize database locality and access patterns, provide adequate DRAM, reduce unnecessary paging and monitor page-fault rates. This reduces access delays and improves concurrent query throughput.

3. Autonomous Vehicle – SRAM, DRAM and MRAM

| Memory | Latency | Capacity | Volatility | Role |
| --- | --- | --- | --- | --- |
| SRAM | Very low | Small | Volatile | Fast cache/buffer |
| DRAM | Low | Large | Volatile | Main sensor-data memory |
| MRAM | Low | Medium | Non-volatile | Persistent/fast memory |

Recommended hierarchy: processor/accelerator registers → SRAM caches and high-speed buffers → high-bandwidth DRAM for camera, LiDAR and radar data → MRAM for suitable non-volatile data and persistent state.

• Use SRAM for the most latency-sensitive sensor-processing buffers.

• Use DRAM for large sensor streams and intermediate AI data because of its capacity and bandwidth.

• Use MRAM where non-volatility and low-power persistent storage are useful.

Result: The hierarchy places time-critical data in fast SRAM and large sensor datasets in DRAM, balancing latency, bandwidth, capacity and power requirements.

4. Edge AI – NAND Flash, DRAM and RRAM

| Memory | Speed | Capacity | Persistence | AI Role |
| --- | --- | --- | --- | --- |
| NAND Flash | Slower than DRAM | High | Non-volatile | Model/storage |
| DRAM | Fast | High | Volatile | Runtime tensors |
| RRAM | Fast potential | High density | Non-volatile | Persistent AI memory |

Recommended hierarchy: AI accelerator/CPU cache → DRAM → NAND Flash, with RRAM used where available for fast persistent model storage or memory-centric AI workloads.

• Store the complete ML model persistently in NAND Flash or suitable RRAM.

• Load frequently used model sections into DRAM before inference.

• Keep active weights, intermediate tensors and working buffers in fast memory.

• Use model quantization and efficient data movement to reduce energy use and improve inference speed.

Result: DRAM handles fast runtime computation, while NAND Flash/RRAM provides persistent model storage and reduces repeated loading overhead.

5. Gaming Console – L3 Cache and DRAM

L3 cache provides lower-latency access than DRAM but has much smaller capacity. Frequently accessed game instructions, data and working-set portions can benefit from L3. Large textures, audio and environment assets are generally too large for cache and therefore depend on DRAM and storage.

• Keep frequently reused game data in L3 through good locality and cache-friendly data structures.

• Provide high DRAM bandwidth so large textures and scene data can be streamed quickly.

• Use prefetching carefully for predictable game data.

• Use efficient asset streaming and decompression so scene transitions do not block rendering.

• Reduce unnecessary memory copies between storage, DRAM and graphics resources.

Recommended hierarchy: CPU/GPU caches → L3/shared cache → high-bandwidth DRAM → persistent storage.

Result: Reducing cache misses and improving DRAM throughput helps maintain smoother frame rendering during large scene transitions.

Overall Conclusion

The proposed memory hierarchies use cache locality, suitable memory technologies, paging optimization and hierarchical organization to reduce latency, improve bandwidth and increase throughput for mobile, cloud, autonomous-vehicle, edge-AI and gaming applications.